MCRPCIe/MCRPCI66 Monitor Documentation

document version 1.0

Copyright © 2012

Masterclock, Inc.

### Introduction

The MCRPCIe/MCRPCI66 Monitor sample application provides the user with a Visual Studio 2010 C++ project which communicates with the Masterclock MCRPCIe/MCRPCI66 service.

The Masterclock McrSync (or McrSync_x64) service communicates with the MCRPCIe/MCRPCI66 using a virtual COM port driver. This driver is installed by Windows when the Masterclock card is installed. When the MCRPCIe/MCRPCI66 is locked to a time code input (SMPTE, IRIG-B, IRIG-B1) or a GPS signal the MCRPCIe/MCRPCI66 service accurately sets the PC time to within +/- 2 msec of the time reference. If there is no reference lock, the service continues to set the PC time to within +/- 2 msec of the TCXO real time clock chip on the MCRPCIe/MCRPCI66 card. If a High Stability Oscillator (HSO) is installed, the MCRPCIe/MCRPCI66 will use the HSO as it’s time reference when there is no reference lock.

The MCRPCIe/MCRPCI66 Monitor sample application reads the MCRPCIe/MCRPCI66 status information using the McrSync (or McrSync_x64) service and displays the status in a dialog box. The project has 32-bit and 64-bit versions. It is important to use the 32-bit or the 64-bit version based on the type of operating system due to registry issues. See additional information later in this document. It is compatible with Windows XP, Windows 7, Server 2003, and Server 2008.

### Abbreviations Used

The following abbreviations are used in this documentation:

API – The application programming interface services provided by the McrSync.dll (or McrSync_x64.dll) library

MCRPCIe/MCRPCI66 – Refers to the MCRPCIe/MCRPCI66 time code reader (TCR), GPS, or oscillator (OSC) card

SMPTE – Society of Motion Picture & Television Engineers-defined time code format

IRIG – Range Commander’s Council/Interrange Instrumentation Group-defined time code format

Windows – One of the Microsoft Windows operating systems (XP,Windows 7,Server 2003,Server 2008)

Driver – The MCRPCIe/MCRPCI66 card driver for the Windows operation system.

Suggested reading:

- SMPTE 12M - Time and Control Code Standard

- SMPTE 309M - Time and Control Code Standard

- Range Commander’s Council IRIG Standard 200-95 – Serial Time Code Formats

- IEEE 1344-1995 – Time zone & date encoding for IRIG-B and IRIG-B1 time codes

- LeitchTM Application Note – Time Code Tutorial-Longitudinal SMPTE Time Code for NTSC Color Video Systems

#### Legal Information

The information contained in this document is subject to change without notice.  Masterclock, Inc. (hereinafter Masterclock) provides the MCRPCIe/MCRPCI66 Monitor sample application on an as-is basis. Masterclock makes no warranty of any kind with regard to this material, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose. Masterclock shall not be liable for errors contained herein or for incidental or consequential damages in connection with the furnishing, performance, or use of this material. The MCRPCIe/MCRPCI66 Monitor sample application is open source. The user is allowed to modify and/or copy code from the application for use with a Masterclock MCRPCIe/MCRPCI66.

### Technical Support

Technical support is limited, except on a custom basis. General comments or bug reports concerning the API package may be addressed, via Internet e-mail, to <support@masterclock.com>.

### API/Document Revision History

- Initial release July 27, 2012.

Use of API Limitation due to Masterclock MCRPCIE/MCRPCI66 service

Only one application at a time is allowed to load the McrSync.dll (for x64 system, McrSync_x64.dll). The Masterclock MCRPCIe/MCRPCI66 service, McrSync (or McrSync_x64), loads the McrSync.dll (or McrSync_x64.dll). If the Masterclock service is running the user CANNOT load the the McrSync.dll (or McrSync_x64.dll). Doing so will cause the Masterclock McrSync (or McrSync_x64) service (and probably the user application) to fail. If the user wishes to monitor the MCRPCIe/MCRPCI66 status, time code status, current time, etc., the user must query the Masterclock service to get this information and NOT use the .dll API calls. The MCRPCIe/MCRPCI66 Monitor sample application shows how to monitor the MCRPCIe/MCRPCI66 via the Masterclock service using a construct called pipes.

### 32-bit versus 64-bit applications

### In general, 32-bit Microsoft applications will run without any issues on 64-bit operating systems. An exception is registry access. For 32-bit applications running on 64-bit operating systems, Microsoft will create Wow6432Node in HKEY_LOCAL_MACHINE/SOFTWARE. The 32-bit applications’ registry accesses to HKEY_LOCAL_MACHINE/SOFTWARE automatically get routed to this folder. What this means is when a 32-bit application accesses HKEY_LOCAL_MACHINE/SOFTWARE, it is actually accessing HKEY_LOCAL_MACHINE/SOFTWARE/Wow6432Node. Therefore, a 32-bit access to a registry key in HKEY_LOCAL_MACHINE/SOFTWARE WILL NOT get the same value as a 64-bit access to a key of the same name. Some of the Masterclock applications use the registry. For Masterclock applications which interact with each other via the registry, it is recommended to always use 64-bit versions on 64-bit operating systems, and 32-bit versions on 32-bit operating systems. An example of this would be a configuration utility which sets registry entries used by a service.

Theory of Operation

The monitor application is a C++ project built using Visual Studio 2010. To use with older versions of Visual Studio the user must create their own project and copy the C++ files and dialog objects into their application. Pipes and other constructs used in this project are compatible with older versions of Visual Studio.

This application assumes the Masterclock MCRPCIe/MCRPCI66 service is installed and running, If debugging, a log file is created (C:\Temp\MCRDEBUG.TXT). The Temp directory must already exist. The user may select the type of information placed in the log file by changing the defines in DebugOutput.h . The instance of the application is created in CMcrMonitorApp::InitInstance() . The Masterclock MCRPCIe/MCRPCI66 service, McrSync (or McrSync_x64), has a named pipe server implemented. To communicate with the MCRPCIe/MCRPCI66 service this monitor application uses the following Masterclock functions: PipeConnect(),PipeTransaction(), and PipeShutdown(). These functions use file commands to interact with the named pipe server in the Masterclock MCRPCIe/MCRPCI66 service. It is strongly suggested the user closely follows the example in these functions when interacting with the MCRPCIe/MCRPCI66 service. For more information on using pipes in Windows see http://msdn.microsoft.com/en-us/library/aa365780(v=VS.85).aspx