MCRPCIe/MCRPCI66 SDK Documentation

document version 1.0

Copyright © 2012 Masterclock, Inc.

### Introduction

The MCRPCIe/MCRPCI66 Software Development Kit (SDK) is a Windows dynamic-link library (DLL) which defines an application programming interface (API) so the DLL can be used. The SDK for the MCRPCIe/MCRPCI66 card driver is compatible with Windows XP, Windows 7, Server 2003, and Server 2008. The DLL comes in two versions, one for x32 systems and one for x64 systems. It is important to use the 32-bit or the 64-bit version based on the type of operating system due to registry issues. See additional information later in this document. To operate, the SDK requires an installed and correctly functioning driver for the MCRPCIe/MCRPCI66 time code reader (TCR), GPS, or oscillator (OSC) card.

The MCRPSIe/MCRPCI66 card supports a GPS receiver, time code reader (TCR), oscillator (OSC), and a High Stability Oscillator (HSO). For time code input, SMPTE (all formats) and IRIG (B0 and B1 formats) time code are supported. Date support is also provided for SMPTE (LeitchTM format or 309M) and IRIG (IEEE 1344). The SDK also provides access to all user groups (SMPTE) and control functions (IRIG) for applications that require access to special information in these fields. The SDK also provides a callback mechanism for MCRPCIe/MCRPCI66 interrupts, where applications can define user-level routines that will be executed when the interrupts occur. While the SDK does not support NMEA messages from the GPS receiver, the end user can configure which NMEA messages are to be output and the COM port they are to be sent to using the Masterclock “MCR Sync Config” configuration utility.

The SDK supports two methods of obtaining time from the MCRPCIe/MCRPCI66. The MCR_ApiPollTimeCode() function provides a simplistic but less accurate method of obtaining time when the input reference is time code. The accuracy of a polled time code is plus or minus the frame rate (e.g. for SMPTE 30 frames/second time code, ± 33ms). The SDK also provides an asynchronous time notification mechanism whereby a user-supplied callback function is called when significant events occur during time code decoding operation. These functions can be used with or without a reference lock (time code or GPS). The time provided with such events is time-stamped with the Windows high-performance timer at the time the event occurs. The timestamp can then be used to derive precise real-time information (+/- 2 millesecond when locked to a reference). An HSO allows accuracy to be maintained during periods when lock to a reference is lost.

### Abbreviations Used

The following abbreviations are used in this documentation:

API – The application programming interface services provided by the McrSync.dll (or McrSync_x64.dll) library.

MCRPCIe/MCRPCI66 – Refers to the MCRPCIe/MCRPCI66 time code reader (TCR), GPS, or oscillator (OSC) card.

SMPTE – Society of Motion Picture & Television Engineers-defined time code format

IRIG – Range Commander’s Council/Interrange Instrumentation Group-defined time code format

Windows – One of the Microsoft Windows operating systems (XP,Windows 7,Server 2003,Server 2008)

Driver – The MCRPCIe/MCRPCI66 card driver for the Windows operation system.

Recommending reading:

- SMPTE 12M - Time and Control Code Standard

- SMPTE 309M - Time and Control Code Standard

- Range Commander’s Council IRIG Standard 200-95 – Serial Time Code Formats

- IEEE 1344-1995 – Time zone & date encoding for IRIG-B and IRIG-B1 time codes

- LeitchTM Application Note – Time Code Tutorial-Longitudinal SMPTE Time Code for NTSC Color Video Systems

#### Legal Information

The information contained in this document is subject to change without notice.  Masterclock, Inc. (hereinafter Masterclock) provides the API development software package on an as-is basis. Masterclock makes no warranty of any kind with regard to this material, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose. Masterclock shall not be liable for errors contained herein or for incidental or consequential damages in connection with the furnishing, performance, or use of this material.

### Technical Support

Technical support is limited, except on a custom basis. General comments or bug reports concerning the API package may be addressed, via Internet e-mail, to <support@masterclock.com>.

### API/Document Revision History

- Initial release July 27, 2012

### Project Integration Instructions

A Visual Studio application will link to McrSync.dll (for x64 system, McrSync_x64.dll). The McrSync.dll (or McrSync_x64.dll) file was created using Visual Studio 2010. It is an unmanaged .dll . For the users application to load the .dll the user may have to load the .dll dynamically. This is especially true if the user application is managed. See http://msdn.microsoft.com/en-us/library/ms810279.aspx for loading a .dll dynamically. Any modules of a Visual C++ application using the API functions should include IntApi.h and IntApiDefs.h .

Use of API Limitation due to Masterclock MCRPCIe/MCRPCI66 service

Only one application at a time is allowed to load the McrSync.dll (or McrSync_x64.dll). The Masterclock MCRPCIe/MCRPCI66 service, McrSync (or McrSync_x64), loads the McrSync.dll (or McrSync_x64.dll). If the Masterclock service is running the user CANNOT load the McrSync.dll (or McrSync_x64.dll). Doing so will cause the Masterclock McrSync (or McrSync_x64) service (and probably the user application) to fail. If the user wishes to monitor the MCRPCIe/MCRPCI66 status, time code status, current time, etc., the user must query the Masterclock service to get this information and NOT use the .dll API calls. The MCRPCIe/MCRPCI66 Monitor sample application shows how to monitor the MCRPCIe/MCRPCI66 via the Masterclock service using a construct called pipes.

### MCRPCIe/MCRPCI66 DLL API Function Reference

MCR_ApiGetDLLInitStatus

The MCR_ApiGetDLLInitStatus returns the DLL internal initialization status.

DWORD MCR_ApiGetDLLInitStatus(void)

##### Parameters

This function has no parameters.

##### Return Values

The return value is one of DLL_INIT_xxx defined in IntApiDefs.h:

| DLL_INIT_OK | The DLL initialized successfully. |
| --- | --- |
| DLL_INIT_UNEXPECTED | Unexpected initialization failure. Contact technical support. |

##### Remarks

The DLL initializes an instance of itself one time. Only one process should load or access the DLL. If the Masterclock service is running, McrSync (or McrSync_x64), the end user SHOULD NOT load the DLL. See “Use of API Limitation due to Masterclock MCRPCIe/MCRPCI66 service”.

MCR_ApiGetTimeCodeStatus (not recommended)

The MCR_ApiGetTimeCodeStatus returns the status of time code decoding on the card. This is a deprecated function. It is recommended to use the MCR_ApiGetLockStatus() function instead which will also return the GPS lock status.

DWORD MCR_ApiGetTimeCodeStatus(void)

##### Parameters

This function has no parameters.

##### Return Values

The return value is one of DLL_TC_STATUS_xxx defined in IntApi.h, currently:

| DLL_TC_STATUS_NORMAL | Time code is available. |
| --- | --- |
| DLL_TC_STATUS_INVALID | Never had a lock to time code |
| DLL_TC_STATUS_LOST | Locked to time code at least once, lock is now lost. |
| DLL_TC_STATUS_CALIB | The MCRPCIe/MCRPCI66 card is calibrating the time code signal level. |

##### Remarks

The time code status is determined by the card driver software and provided to the DLL for reporting purposes.

MCR_ApiGetLockStatus

The MCR_ ApiGetLockStatus returns the lock status information of the various functions on the MCRPCIe/MCRPCI66 card. This includes the overall lock status, time code lock status, GPS lock status, HSO lock status, and HSO calibration achievement level.

void MCR_ApiGetLockStatus(API_LAST_LOCK_STATUS *pLockStatus)

##### Parameters

This function has no parameters.

##### Return Values

Modifies the data in the pLockStatus structure for the current status defined in IntApiDefs.h,

| pLockStatus->ucLockStatus | LOCK_INVALID LOCK_NOT_PRESENT LOCK_LOST LOCK_RESTORED LOCK_PRESENT |
| --- | --- |
| pLockStatus->ucGpsStatus | LOCK_INVALID LOCK_NOT_PRESENT LOCK_LOST LOCK_RESTORED LOCK_PRESENT |
| pLockStatus->ucTimeCodeStatus | LOCK_INVALID LOCK_NOT_PRESENT LOCK_LOST LOCK_RESTORED LOCK_PRESENT |
| pLockStatus->ucHsoStatus | LOCK_INVALID LOCK_NOT_PRESENT LOCK_LOST LOCK_RESTORED LOCK_PRESENT |
| pLockStatus->ucHsoAchievementLevel | When locked to GPS the HSO frequency undergoes continuous calibration. This number indicates the level of the calibration. A zero indicates no calibrating active. Value 1-4, initial calibration. Value 5-15 current calibration level. |
| pLockStatus->ucRtcStatus | LOCK_INVALID LOCK_NOT_PRESENT LOCK_LOST LOCK_RESTORED LOCK_PRESENT |

##### Remarks

ucLockStatus contains the overall lock status and should be used instead of the individual lock statuses.

MCR_ApiGetDeviceStatus

The MCR_ApiGetDeviceStatus function returns the status of communication with the MCRPCIe/PCRPCI66.

DWORD MCR_ApiGetDeviceStatus(void)

##### Parameters

This function has no parameters.

##### Return Values

The return value is one of HW_STATUS_xxx defined in IntApi.h, currently:

| HW_STATUS_NORMAL | Driver is interacting normally with the MCRPCIe/PCRPCI66. |
| --- | --- |
| HW_STATUS_CARDCOMMFAIL | Driver is failing I/O port communications with the MCRPCIe/PCRPCI66. |

##### Remarks

The last asynchronous or polled time code operation sets this status.

MCR_ApiSetFrameRate (deprecated)

This function is deprecated for the MCRPCIe/MCRPCI66 and does nothing. Since the MCRPCIe/MCRPCI66 detects the frame rate, this function is no longer necessary. It still exists, but the frame rate is set from the incoming time code data, not by this function.

MCR_ApiPollTimeCode

The MCR_ApiPollTimeCode returns the current time code status.

DWORD MCR_ApiPollTimeCode(

PTCRTIME ptcrt // pointer to TCRTIME structure that receives polled time data

);

##### Parameters

tcrt

Pointer to a TCRTIME structure that is to receive polled time data.

##### Return Values

The return value is one of DLL_TC_STATUS_xxx defined in IntApi.h:

| DLL_TC_STATUS_NORMAL | Time code is available. |
| --- | --- |
| DLL_TC_STATUS_INVALID | Time code data is out of range. |
| DLL_TC_STATUS_LOST | Time code is not available. |
| DLL_TC_STATUS_DRVFAIL | Time code not available because of failed communications with the MCRPCIe/PCRPCI66 driver. |
| DLL_TC_STATUS_CALIB | The MCRPCIe/MCRPCI66 card is calibrating to the time code signal level. |

##### Remarks

The API will attempt to fill the date members of the TCRTIME structure relative to the type of time code being decoded.  If the time code source is not actually encoding date, the value of the date members is undefined.  It is the responsibility of the application programmer to determine if date will or will not be encoded by the time code source.

MCR_ApiEnableCallback

The MCR_ApiEnableCallback function enables the asynchronous callback of a user-supplied callback function, or adds hooks to an already-established callback function, on behalf of the calling thread.

DWORD MCR_ApiEnableCallback(

DWORD cbToAdd, // mask of callback hooks

PTCRASYNC_CALLBACK_ROUTINE cbFunction // pointer to user-supplied callback function

);

##### Parameters

cbToAdd

Any combination of callback hooks ASYNC_xxx (defined in IntApi.h) to be enabled or added, from:

| ASYNC_TIMESET | Once/minute interrupt (second = 30, frame = 0) |
| --- | --- |
| ASYNC_PERSEC | Once/second interrupt (second = 0) |
| ASYNC_ENDTC | Once/frame interrupt (after end of time code frame) |
| ASYNC_TIMELOSS | Time code lost |
| ASYNC_TIMEGAIN | Time code regained from lost state |

cbFunction

A user-supplied callback function of type PTCRASYNC_CALLBACK_ROUTINE. A skeleton example of the callback function would be:

DWORD _stdcall MCR_ApiAsyncCallback(DWORD dwStatus, PVOID data, DWORD dataLen)

{

// code here to handle asynchronous notifications

}

##### Return Values

On success, the new callback hooks mask (always non-zero).  On failure, 0.

##### Remarks

Each thread in a process may have an active callback function but no more than one. Additional calls to MCR_ApiEnableCallback in the thread will use the function passed by the initial call. All calls to MCR_ApiEnableCallback are on behalf of the calling thread, not the calling process. MCR_ApiEnableCallback  must be called once for every event that the callback function should be run for. Enabling a callback, in some circumstances, can affect the callback hooks of other threads or other processes using the API.  See the section entitled Relationship of MCRPCIe/PCRPCI66 Interrupts to API Asynchronous Notifications for additional description of issues regarding the use of this function.

MCR_ApiDisableCallback

The MCR_ApiDisableCallback function disables one or more of the callback hooks currently active for the

callback function of the calling thread.

DWORD MCR_ApiDisableCallback(

DWORD cbToRemove // mask of callback hooks to remove

);

##### Parameters

cbToRemove

Any combination of callback hooks ASYNC_xxx (defined in IntApiDefs.H) to be disabled for the callback

function in use by the calling thread:

| ASYNC_TIMESET | Once/minute interrupt (second = 30, frame = 0) |
| --- | --- |
| ASYNC_PERSEC | Once/second interrupt (second = 0) |
| ASYNC_ENDTC | Once/frame interrupt (after time code data, before sync word) |
| ASYNC_TIMELOSS | Time code lost |
| ASYNC_TIMEGAIN | Time code regained from lost state |

##### Return Values

On success, the new callback hooks mask

On failure, ASYNC_MASK_ERROR

##### Remarks

A thread must disable its callback function before terminating.

Each thread in a process may have an active callback function but no more than one. All calls to MCR_ApiDisableCallback are on behalf of the calling thread, not the calling process. Disabling a callback hook does not affect the callback hooks of other threads or other processes.  Call MCR_ApiDisableCallback with cbToRemove set to ASYNC_ALL to disable the callback function for the current thread. When all callback hooks have been disabled for a callback function the function is no longer called and is removed from the API’s internal callback tables.  See the section entitled Relationship of MCRPCIe/PCRPCI66 Interrupts to the API Asynchronous Notification for additional issues regarding the use of this function.

MCR_ApiShutdown (deprecated)

For the MCRPCIe/MCRPCI66, this function is now deprecated and does nothing.

MCR_ApiClearEnhanced

The MCR_ApiClearEnhanced function forces off all enabled enhanced interrupt features on the MCRPCIe/MCRPCI66

time code reader card.

BOOL MCR_ApiClearEnhanced(void)

##### Parameters

This function has no parameters.

##### Return Values

Returns TRUE if all enhanced features cleared successfully.  Returns FALSE if unable to clear enhanced features.

##### Remarks

Use of this function defies the multithread-safe capability of this API and is not recommended for use in an environment where multiple applications will use the asynchronous notifications.

MCR_ApiGetTimeCodeType

The MCR_ApiGetTimeCodeType function returns the time code type currently in use.

DWORD MCR_ApiGetTimeCodeType(void)

##### Parameters

This function has no parameters

##### Return Values

On of TC_TYPE_xxx defined in IntApi.h:

| TC_TYPE_SMPTE | SMPTE time code type |
| --- | --- |
| TC_TYPE_IRIG | IRIG-B time code type |
| TC_TYPE_UNDEFINED | Undefined time code type |

MCR_ApiGetDLLVersion

The MCR_ApiGetDLLVersion function returns the software version of the TCR DLL API library.

void MCR_ApiGetDLLVersion(

PUINT verMaj, // receives major version number

PUINT verMin, // receives minor version number

PUINT verRev // receives revision level

);

##### Parameters

verMaj

A pointer to a variable that will receive the major version of the API library.

verMin

A pointer to a variable that will receive the minor version of the API library.

verRev

A pointer to a variable that will receive the revision level of the API library.

##### Return Values

This function returns no values.

MCR_ApiGetDriverVersion

The MCR_ApiGetDriverVersion function returns the software version of the MCRPCIe/PCRPCI66 driver.

BOOL MCR_ApiGetDriverVersion(

PUINT verMaj, // receives major version number

PUINT verMin, // receives minor version number

PUINT verRev // receives revision level

);

##### Parameters

verMaj

A pointer to a variable that will receive the major version of the MCRPCIe/PCRPCI66 driver.

verMin

A pointer to a variable that will receive the minor version of the MCRPCIe/PCRPCI66 driver.

verRev

A pointer to a variable that will receive the revision level of the MCRPCIe/PCRPCI66 driver.

##### Return Values

Returns TRUE with the MCRPCIe/PCRPCI66 driver version number in parameters.  Returns FALSE if driver version could not be determined.

MCR_ApiGetFirmwareVersion

The MCR_ApiGetFirmwareVersion function returns the software version of the MCRPCIe/PCRPCI66 driver.

BOOL MCR_ApiGetFirmwareVersion(

PUINT verMaj, // receives major version number

PUINT verMin, // receives minor version number

PUINT verRev // receives revision level

);

##### Parameters

verMaj

A pointer to a variable that will receive the major version of the MCRPCIe/PCRPCI66 firmware.

verMin

A pointer to a variable that will receive the minor version of the MCRPCIe/PCRPCI66 firmware.

verRev

A pointer to a variable that will receive the revision level of the MCRPCIe/PCRPCI66 firmware.

##### Return Values

Returns TRUE with the MCRPCIe/PCRPCI66 firmware version number in parameters. Returns FALSE if the firmware version could not be determined.

MCR_ApiGetFirmwareChecksum (deprecated)

The MCR_ApiGetFirmwareChecksum function is deprecated and now does nothing..

MCR_ApiRequestRecalibration

The MCR_ApiRequestRecalibration function forces the MCRPCIe/MCRPCI66 card to recalibrate the automatic gain control to the incoming time code’s signal level.

BOOL MCR_ApiRequestRecalibration (void)

##### Parameters

This function has no parameters.

##### Return Values

Returns TRUE if recalibration started successfully.

##### Remarks

Once the MCRPCIe/MCRPCI66 card has detected the time code format and calibrated its gain level to the time code’s signal level it will expect that level to remain constant. If the time code format changes or the level fluctuates, the card may cease to decode time code properly. In a situation where a level change is anticipated, such as a switch to a different time code feed, this API instructs the MCRPCIe/MCRPCI66 to recalibrate the gain.

This function can be used to switch between SMPTE and IRIG-B time codes.

BOOL MCR_ApiSetDateTime(MCRSYNC_IPC_SETTIME *psrSetTime)

The MCR_ApiSetDateTime sets the time in the Real Time Clock chip (TCXO RTC) on the MCRPCIe/MCRPCI66 card. This time is then used to set the PC time.

##### Parameters

MCRSYNC_IPC_SETTIME *psrSetTime – pointer to the structure containing the time to set. See IntApiDefs.h..

##### Return Values

Returns TRUE if time on the card was set successfully.

##### Remarks

When the service is running, the card is always controlling the PC time. If the PC time is within 100 msec of the card time, skewing occurs. If greater than 100 msec, the PC time is jam synced to the card  time.

UINT32 MCR_ApiSetConfiguration(MCR_CONFIGURATION *pApiSetConfig)

The MCR_ApiSetConfiguration sets the card and the service configuration and saves the settings in the registry. Every time the service is started, the settings in the registry are verified with those on the card and in the service. If a difference is found, the settings in the registry are used.

##### Parameters

MCR_CONFIGURATION *pApiSetConfig – pointer to the structure containing the configuration parameters. See IntApiDesf.h .

##### Return Values

Returns unsigned 32 bit integer, ==0 if the configuration was set successfully, else one or more bits are set returning one or more error flags, see NMEA_MESSAGE_xxx in IntApiDefs.h.

##### Remarks

The configuration only gets saved if no errors are detected.

void MCR_ApiGetConfiguration(MCR_CONFIGURATION *pApiGetConfig)

The MCR_ApiGetConfiguration sets the card and the service configuration and saves the settings in the registry. Every time the service is started, the settings in the registry are verified with those on the card and in the service. If a difference is found, the settings in the registry are used.

##### Parameters

MCR_CONFIGURATION *pApiGetConfig – pointer to the structure containing the configuration parameters. See IntApiDefs.h .

##### Return Values

Returns returns the condfiguration parameters in the structure pointed to by pApiGetConfig. See the MCR_CONFIGURATION structure in  IntApiDefs.h.

##### Remarks

The configuration only gets saved if no errors are detected.

# The Asynchronous Callback Function

When enabled, the API will call a user-supplied callback function reporting the activation of interrupts on the card.

An example of the callback function in C/C++:

DWORD _stdcall MCR_ApiAsyncCallback (DWORD dwStatus, PVOID data, DWORD dataLen)

{

PTCRTIME time;

switch(dwStatus)

{

case ASYNC_PERSEC:

// data is a pointer to a TCRTIME data structure and dataLen = sizeof(TCRTIME)

time = (PTCRTIME)data;

break;

case ASYNC_TIMESET:

// data is a pointer to a TCRTIME data structure and dataLen = sizeof(TCRTIME)

time = (PTCRTIME)data;

break;;

case ASYNC_ENDTC:

// data is a pointer to a TCRTIME data structure and dataLen = sizeof(TCRTIME)

time = (PTCRTIME)data;

break;

case ASYNC_TIMELOSS:

// data is NULL and dataLen = 0

break;

case ASYNC_TIMEGAIN:

// data is NULL and dataLen = 0

break;

}

return(0);

}

The callback function may have any name that the user application requires, but must have the exact calling convention and parameter declaration as shown above to avoid corruption of the stack when the function is called by the API.

When the callback function provides a TCRTIME data structure as the data passed, the date members of the TCRTIME structure will be set relative to the type of time code being decoded.  If the time code source is not actually encoding date, the value of the date members is undefined.  It is the responsibility of the application programmer to determine if date will or will not be encoded by the time code source.

### Structure and Constants Reference

// A structure defining a SMPTE time code frame (the union ‘rawTc’ member of TCRTIME will contain

// this data structure if operating with SMPTE time code.

typedef struct _SMPTE

{

UCHAR fu; // frame units

UCHAR u1; // user group 1

UCHAR ft; // frame tens

UCHAR u2; // user group 2

UCHAR su; // second units

UCHAR u3; // user group 3

UCHAR st; // second tens

UCHAR u4; // user group 4

UCHAR mu; // minute units

UCHAR u5; // user group 5

UCHAR mt; // minute tens

UCHAR u6; // user group 6

UCHAR hu; // hour units

UCHAR u7; // user group 7

UCHAR ht; // hour tens

UCHAR u8; // user group 8

} SMPTE, *PSMPTE;

// A structure defining an IRIG-B or IRIG-B1 time code frame (the union ‘rawTc’ member of TCRTIME will contain

// this data structure if operating with IRIG-B or IRIG-B1 time code.

typedef struct _IRIG

{

UCHAR su; // second units

UCHAR st; // second tens

UCHAR mu; // minute units

UCHAR mt; // minute tens

UCHAR hu; // hour units

UCHAR ht; // hour tens

UCHAR du; // day units

UCHAR dt; // day tens

UCHAR dh; // day hundreds

UCHAR cf1; // control functions 1 (see card manual for more information)

UCHAR cf2; // control functions 2 (see card manual for more information)

UCHAR cf3; // control functions 3 (see card manual for more information)

UCHAR cf4; // control functions 4 (see card manual for more information)

UCHAR cf5; // control functions 5 (see card manual for more information)

UCHAR cf6; // control functions 6 (see card manual for more information)

UCHAR cf7; // control functions 7 (see card manual for more information)

} IRIG, *PIRIG;

// TCRTIME data structure provided by polled and asynchronous time responses

typedef struct _TCRTIME

{

// What type of timecode are we decoding

DWORD dwTimeCodeType;

// raw time code values

union

{

IRIG 		irig;

SMPTE 	smpte;

} rawTc;

// if a time given asynchronously (by an interrupt on MCRPCIe/PCRPCI66), contains high-performance counter

// reading at time of interrupt, for polled time codes the value of this member is undefined

LARGE_INTEGER intPerfCount;

// decoded time, remember that wMilliseconds is not provided unless frame rate is set

// via TCRSetFrameRate() call.

WORD 		wHour;

WORD 		wMinute;

WORD 		wSecond;

WORD 		wMilliseconds;

WORD 		wMonth;

WORD 		wDay;

WORD 		wYear;

// validity flag (does checksum pass?  Deprecated, not used)

BYTE		valid;

} TCRTIME, *PTCRTIME;

// Communication statuses between the device driver and MCRPCIe/MCRPCI66

HW_STATUS_NORMAL 		0x00 // normal

HW_STATUS_CARDCOMMFAIL 	0x01 // I/O communication failure

// Time code types

TC_TYPE_SMPTE		0x00000001L

TC_TYPE_IRIG			0x00000002L	// deprecated

TC_TYPE_IRIG_B1		0x00000002L	// originally, IRIG was only B1

TC_TYPE_IRIG_B		0x00000003L

TC_TYPE_UNDEFINED	0x00008000L

// DLL time code statuses

DLL_TC_STATUS_NORMAL 	0x00 	// time code read was successfully

DLL_TC_STATUS_INVALID 	0x01 	// time code data is invalid

DLL_TC_STATUS_LOST 	0x02 	// time code not available

DLL_TC_STATUS_DRVFAIL 	0x03 	// driver communication failure

DLL_TC_STATUS_CALIB	0x04	// time code signal calibration in progress

// DLL initialization statuses

DLL_INIT_OK 			0	// initialization was successful

DLL_INIT_CFGERR 		3 	// configuration is invalid/missing

DLL_INIT_NOKMD 		4 	// unable to find the driver

DLL_INIT_TCPROBLEM	5	// unable to detect time code type

DLL_INIT_INV_DLL		6	// problem loading DLL or its functions

DLL_INIT_UNEXPECTED 	255 	// unexpected failure

// Asynchronous notifications callback hooks (maps to interrupts)

ASYNC_TIMESET 		0x00000001	// once/minute (sec=30, frame=0)

ASYNC_TIMEGAIN 		0x00000002 	// time code regained from lost state

ASYNC_TIMELOSS 		0x00000004 	// time code lost interrupt

ASYNC_ENDTC 		0x00000008 	// 1/frame (after time code/before sync word)

ASYNC_STARTTC 		0x00000010 	// 1/frame (before time code, after sync word)

ASYNC_PERSEC 		0x00000020 	// once/second interrupt (frame=0)

ASYNC_ALL 			0x7FFFFFFF

ASYNC_MASK_ERROR 	0x80000000

#define INVALID_DST_ENCODED           		0x00000001UL

#define INVALID_FREEWHEEL_DISPLAY     		0x00000002UL

#define INVALID_FREEWHEEL_SYNCHRONIZE 	0x00000004UL

#define INVALID_TIME_CODE_LOSS_NOTIFY 		0x00000008UL

#define INVALID_TIME_CODE_SOURCE      		0x00000010UL

#define INVALID_TIME_ZONE_BIAS        		0x00000020UL

#define INVALID_STANDARD_BIAS         		0x00000040UL

#define INVALID_STANDARD_YEAR         		0x00000080UL

#define INVALID_STANDARD_MONTH        		0x00000100UL

#define INVALID_STANDARD_DAY_OF_WEEK  	0x00000200UL

#define INVALID_STANDARD_DATE         		0x00000400UL

#define INVALID_STANDARD_HOUR         		0x00000800UL

#define INVALID_STANDARD_MINUTE       		0x00001000UL

#define INVALID_DAYLIGHT_BIAS         		0x00002000UL

#define INVALID_DAYLIGHT_YEAR         		0x00004000UL

#define INVALID_DAYLIGHT_MONTH        		0x00008000UL

#define INVALID_DAYLIGHT_DAY_OF_WEEK  	0x00010000UL

#define INVALID_DAYLIGHT_DATE         		0x00020000UL

#define INVALID_DAYLIGHT_HOUR         		0x00040000UL

#define INVALID_DAYLIGHT_MINUTE       		0x00080000UL

#define INVALID_NMEA_CONFIGURATION    		0x00100000UL

// bitmap for enabled NMEA messages

#define NMEA_MESSAGE_MASK	0x7F

#define NMEA_MESSAGE_BIT_ZDA	0x01

#define NMEA_MESSAGE_BIT_GSV	0x02

#define NMEA_MESSAGE_BIT_GGA	0x04

#define NMEA_MESSAGE_BIT_GLL	0x08

#define NMEA_MESSAGE_BIT_GSA	0x10

#define NMEA_MESSAGE_BIT_RMC	0x20

#define NMEA_MESSAGE_BIT_VTG	0x40

#define NMEA_MESSAGE_BIT_GST	0x80

#define NMEA_PARITY_NONE	0

#define NMEA_PARITY_ODD	1

#define NMEA_PARITY_EVEN	2

typedef struct {

INT32 DstEncoded;		// ==0, time code never offset by daylight time

// ==1, time code offset by daylight time during daylight period

INT32 FreewheelDisplay;	// ==0, time code does not contain Masterclock, Inc proprietary

//freewheeling indicator in SMPTE frame

// ==1, when freewheeling, time code will indicate such using

//Masterclock, Inc proprietary lock indicator in SMPTE frame

INT32 FreewheelSynchronize;	// ==0, when freewheeling (FreewheelDisplay must ==1 to know

//when freewheeling), do not set system time

// ==1, when freewheeling (FreewheelDisplay must ==1 to know

//when freewheeling), set system time to incoming time code

INT32 TimeCodeLossNotify;	// ==0, do not generate alarm when time code lock is lost

// ==1, generate alarm when time code lock is lost

INT32 PciComPort;        	// serial port number to use

// settings for outputting the NMEA messages

INT32 iNmeaMessagesBitMap;	// See NMEA_MESSAGE_xxx above

INT32 iNmeaBaudrate;		// 4800, 9600, 19200, 38400, 57600, 115200

INT32 iNmeaDataBits;		// 7, 8

INT32 iNmeaParity;		// 0, 1, 2 (see NMEA_PARITY_xxx above)

INT32 iNmeaStopBits;		// 1,2

INT32 iNmeaComPortNumber;

INT32 TimeCodeSourceSmpte;	// see TC_SOURCE_DATE_NOT_INCLUDED, etc.

INT32 TimeCodeSourceIrig;	// see TC_SOURCE_DATE_NOT_INCLUDED, etc.

INT32 TimeZoneBiasSeconds;	// time zone bias, in seconds

INT32 StandardBiasSeconds;	// standard bias, in seconds (usually zero)

INT32 StandardYear;		// zero for relative standard time, else year for absolute standard

// date (usually zero)

INT32 StandardMonth;		// month standard time begins

INT32 StandardDayOfWeek;	// day of week standard time begins, SUN==1, MON==2,

// TUE==3, etc.

INT32 StandardDate;		// date of month standard time begins

INT32 StandardHour;		// hour of day standard time begins

INT32 StandardMinute;		// minute of hour standard time begins

INT32 DaylightBiasSeconds;	// daylight bias, in seconds (usually zero)

INT32 DaylightYear;		// zero for relative daylight time, else year for absolute daylight

// date (usually zero)

INT32 DaylightMonth;		// month daylight time begins

INT32 DaylightDayOfWeek;	// day of week daylight time begins, SUN==1, MON==2,

// TUE==3, etc.

INT32 DaylightDate;		// date of month daylight time begins

INT32 DaylightHour;		// hour of day daylight time begins

INT32 DaylightMinute;		// minute of hour daylight time begins

} MCR_CONFIGURATION;

// User callback routine function type

typedef DWORD (__stdcall *PTCRASYNC_CALLBACK_ROUTINE)(DWORD dwStatus, PVOID data,

DWORD dataLen);

### Relationship of MCRPCIe/PCRPCI66 Interrupts to API Asynchronous Notifications

This section describes the relationship between the interrupts that the MCRPCIe/PCRPCI66 throws in response to time code events and the API asynchronous notification hooks. The MCRPCIe/PCRPCI66 driver is an intermediary between the aforementioned objects but this is transparent to the application programmer. Knowledge of the information in this section is not required to use the API but should be considered in scenarios where multi-process/thread use of API will be implemented.

| ##### MCRPCIe/PCRPCI66 interrupt | ##### API callback hook |
| --- | --- |
| Time Set Interrupt (Once/minute interrupt thrown when time code seconds= 30, frame = 0) | ASYNC_TIMESET |
| Once/Second Interrupt (Once/second interrupt thrown when time code frame = 0) | ASYNC_PERSEC |
| End Time Code Interrupt (Once/frame interrupt thrown after time code data decoded, before sync word) | ASYNC_ENDTC |
| Time Code Loss (Thrown when time code is lost from an active decoding state) | ASYNC_TIMELOSS |
| Time Gain Interrupt (Thrown when time code is regained from a lost state) | ASYNC_TIMEGAIN |

The Time Set Interrupt, Time Loss Interrupt, and Time Gain Interrupt are always thrown by the MCRPCIe/PCRPCI66 when said event occurs and cannot be disabled. These interrupts are supported by the MCRPCIe/PCRPCI66 in all modes of time code decoding operation.

When the Once/Second Interrupt is enabled the Time Set Interrupt is preempted and will not be passed by the MCRPCIe/PCRPCI66 driver to the API. When the Start Time Code Interrupt is enabled both the Time Set Interrupt and Once/Second Interrupt are preempted. The reason for the preemption is that all three interrupts, regardless of frequency, will fall at the same time code transition point in real-time. The preemption is internal to the MCRPCIe/PCRPCI66.  When a thread enables an asynchronous notification callback hook that maps to a MCRPCIe/PCRPCI66 “enhanced interrupt” that interrupt will be enabled on time code reader card. It follows that one can break the operation of another thread/process’s callback functions by enabling an “enhanced interrupt” on the MCRPCIe/PCRPCI66 that preempts another interrupt (as previously discussed). Caution should be exercised in planning what callback hooks will be used by an application calling the API’s. In environments where multithreaded/multiprocess use of the API is planned it is strongly recommended that all applications utilize only the ASYNC_TIMESET callback hook (Time Set Interrupt).